1. Voice data structures

// Voice encode mode

// Due to a hardware bug, the actual configuration is the following

// VOICE_ENCODE_PCMA = 0: invalid

// VOICE_ENCODE_PCMU = 1: CVSD

// VOICE_ENCODE_CVSD = 2: PCM-u

// VOICE_ENCODE_BYP = 3: not compressed raw data

// It will be fixed in A2 revision back to the its originally intended configuration

typedef enum {

VOICE_ENCODE_PCMA = 0,

VOICE_ENCODE_PCMU = 1,

VOICE_ENCODE_CVSD = 2,

VOICE_ENCODE_BYP = 3

}VOICE_ENCODE_t;

// Voice sample rate

typedef enum {

VOICE_RATE_64K = 0,

VOICE_RATE_32K = 1,

VOICE_RATE_16K = 2,

VOICE_RATE_8K = 3

}VOICE_RATE_t;

// Voice notch filter configuration

typedef enum {

VOICE_NOTCH_BYP = 0,

VOICE_NOTCH_1	= 1,

VOICE_NOTCH_2	= 2,

VOICE_NOTCH_3	= 3

}VOICE_NOTCH_t;

// Voice polarity selection

typedef enum {

VOICE_POLARITY_POS = 0,

VOICE_POLARITY_NEG = 1

}VOICE_POLARITY_t;

enum{

HAL_VOICE_EVT_DATA = 1,

HAL_VOICE_EVT_FAIL = 0xff

};

// Voice configuration structure

typedef struct _voice_Cfg_t{

bool						voiceSelAmicDmic;

GPIO_Pin_e 			dmicDataPin;

GPIO_Pin_e 			dmicClkPin;

uint8_t					amicGain;

uint8_t					voiceGain;

VOICE_ENCODE_t	voiceEncodeMode;

VOICE_RATE_t 		voiceRate;

bool						voiceAutoMuteOnOff;

}voice_Cfg_t;

// Voice event structure

typedef struct _voice_Evt_t{

int       type;

uint32_t* data;

uint32_t  size;

}voice_Evt_t;

typedef void (*voice_Hdl_t)(voice_Evt_t* pev);

// Voice context structure

typedef struct _voice_Contex_t{

bool      		enable;

voice_Cfg_t		cfg;

voice_Hdl_t		evt_handler;

}voice_Ctx_t;

2. Voice API functions

// Enable voice core

void hal_voice_enable(void);

// Disable voice core

void hal_voice_disable(void);

// Select DMIC

void hal_voice_dmic_mode(void);

// Select AMIC

void hal_voice_amic_mode(void);

// Open a GPIO pin for DMIC

void hal_voice_dmic_open(GPIO_Pin_e dmicDataPin, GPIO_Pin_e dmicClkPin);

// Set PGA gain for AMIC

void hal_voice_amic_gain(uint8_t amicGain);

// Set voice process gain

void hal_voice_gain(uint8_t voiceGain);

// Set voice encoding mode

void hal_voice_encode(VOICE_ENCODE_t voiceEncodeMode);

// Set voice data rate

void hal_voice_rate(VOICE_RATE_t voiceRate);

// Enable voice auto-mute

void hal_voice_amute_on(void);

// Disable voice auto-mute

void hal_voice_amute_off(void);

// Voice interrupt handler

void __attribute__((weak)) hal_ADC_IRQHandler(void);

// Allocate memory and power manager for voice

void hal_voice_init(void);

// Configure voice capture

int hal_voice_config(voice_Cfg_t cfg, voice_Hdl_t evt_handler);

// Start voice capture

int hal_voice_start(void);

// Stop voice capture

int hal_voice_stop(void);

// Clear memory and power manager for voice

int hal_voice_clear(void);

3. Voice capture application examples

3.1. Variables for voice capture with voice buffer

#define    MAX_VOICE_BUF_SIZE		24576

static uint8 voiceDemo_TaskID;   // Task ID for internal task/event processing

static int voiceWriteID;

static uint32_t voiceBuf[MAX_VOICE_BUF_SIZE];

static int voiceBufIndex;

3.2. Voice event handler with voice buffer, read voice data and write to voice buffer

static void voice_evt_handler_buf(voice_Evt_t *pev)

{

if(pev->type == HAL_VOICE_EVT_DATA){

//		LOG("Type matched\n");

int n;

for (n = 0; n < pev->size; n++) {

voiceBuf[voiceBufIndex] = pev->data[n];

voiceBufIndex++;

}

voiceWriteID++;

LOG("Voice data saved successfully:");

LOG("%d %d\n", voiceWriteID, pev->data[pev->size-1]);

}

}

3.3. Configure and start voice capture task

static void voiceCaptureTask( void )

{

voice_Cfg_t cfg;

cfg.voiceSelAmicDmic = 1;

cfg.dmicDataPin = P4;

cfg.dmicClkPin = P5;

cfg.amicGain = 0;

cfg.voiceGain = 40;

cfg.voiceEncodeMode = VOICE_ENCODE_CVSD;

cfg.voiceRate = VOICE_RATE_8K;

cfg.voiceAutoMuteOnOff = 1;

volatile int voiceConfigStatus = hal_voice_config(cfg, voice_evt_handler_buf);

if(voiceConfigStatus) {

LOG("Voice configuration failed\n");

return;

}

else {

LOG("Voice configuration succeeded\n");

}

LOG("Voice capture started\n");

// Restart timer

osal_start_timerEx( voiceDemo_TaskID, voiceStopTask_EVT, 5000 );

hal_voice_start();

}

3.4. Stop voice capture task

// This function will print the voice buffer data via UART

static void voiceStopTask( void )

{

hal_voice_stop();

LOG("Voice capture stopped\n");

LOG("Voice data word saved:");

LOG("%d\n", voiceBufIndex);

if (hal_uart_get_tx_ready() == PPlus_SUCCESS) {

int voiceTxmitIndex;

voiceTxmitIndex = 0;

while (voiceTxmitIndex < voiceBufIndex) {

LOG("%x\n",voiceBuf[voiceTxmitIndex]);

voiceTxmitIndex++;

}

LOG("Voice data byte transmitted:");

LOG("%d\n", voiceTxmitIndex);

LOG("Voice data buffer transmitted successfully");

}

}

3.5. Initialize voice buffer pointers and start voice capture

void voice_Init( uint8 task_id )

{

voiceDemo_TaskID = task_id;

voiceWriteID = 0;

voiceBufIndex = 0;

voiceCaptureTask();

}

3.6. Voice event process

uint16 voice_ProcessEvent( uint8 task_id, uint16 events )

{

VOID task_id; // OSAL required parameter that isn't used in this function

LOG("voice_ProcessEvent: 0x%x\n",events);

if ( events & SYS_EVENT_MSG )

{

uint8 *pMsg;

if ( (pMsg = osal_msg_receive( voiceDemo_TaskID )) != NULL )

{

voice_ProcessOSALMsg( (osal_event_hdr_t *)pMsg );

// Release the OSAL message

VOID osal_msg_deallocate( pMsg );

}

// return unprocessed events

return (events ^ SYS_EVENT_MSG);

}

if ( events == voiceCaptureTask_EVT )

{

LOG("voiceCaptureTask_EVT\n");

voiceCaptureTask();

return (events ^ voiceCaptureTask_EVT);

}

if ( events == voiceStopTask_EVT )

{

// Perform periodic heart rate task

LOG("voiceStopTask_EVT\n");

voiceStopTask();

return (events ^ voiceStopTask_EVT);

}

// Discard unknown events

return 0;

}

3.7. OSAL initialization for voice applications

const pTaskEventHandlerFn tasksArr[] =

{

LL_ProcessEvent,

voice_ProcessEvent

};

void osalInitTasks( void )

{

uint8 taskID = 0;

tasksEvents = (uint16 *)osal_mem_alloc( sizeof( uint16 ) * tasksCnt);

osal_memset( tasksEvents, 0, (sizeof( uint16 ) * tasksCnt));

LL_Init( taskID++ );

/* Application */

voice_Init( taskID );

}

4. Matlab scripts for captured voice data processing and playback

Process and playback raw voice data: pcmraw.m

Process and playback PCM-u coded voice data: pcmu.m

Process and playback PCM-A coded voice data: pcma.m

Process and playback CVSD coded voice data: cvsd.m